DRAFT NBS Specification

Troldtekt akustik panels for wall cladding installed with concealed KN brackets on wooden battens

(Wall lining)

K11

- WALL LINING

Please refer to Architect’s drawings ……………………. for locations

Substrate: Wooden battens (minimum size 22 x 95mm)

- Additional supports: ………………………

- Wall lining manufacturer/supplier: Troldtekt A/S, Sletvej 2A, 8310 Tranbjerg J, Denmark Tel: +45 8747 8100 Fax +45 8747 8111 Email: sales@troldtekt.co.uk

- Product reference: Troldtekt akustik

- Panel size: 600X600mm, 600X1200mm, 600X2400mm, special size.

- Thickness: 35mm

- Panel finish: Natural wood / Standard white (RAL9003) / other colour

- Structure: Extreme fine 0.5mm, Ultrafine 1mm, Fine 1.5mm

- Edge design: K0-N - Square edges, K5-N - 5mm bevel, K11-N - 11mm bevel.

- Lining type: Fine fibre wood wool panels meeting EN 13168 and EN 13964 standards. The panels must be available as be PEFC™ or FSC®-certified (FSC® C115450). Certified “Silver” Cradle to Cradle” and the "Allergy Friendly Product Award" - "Allergy UK".

- Lining module: 600X600mm, 600X1200mm, 600X2400mm.

- Manufacturer’s Troldtekt components to include KN brackets and Troldtekt screws for wood.

- Insulation:	30 or 50mm stone wool (45kg/m3) acoustic pads.

Setting out

- Long edges horizontal and centred on supports

Fixing to supports

- Fasteners: Troldtekt KN brackets, Troldtekt wood screw 4.2 x 55mm (35mm panels). Installation should be in accordance with manufacturer’s instructions, see www.troldtekt.com for download.

- Fixing: Install battens (minimum size 22x95mm) horizontally with a centre spacing

of 600mm starting from the base of the wall. Set out the Troldtekt panels along the wall base starting from the centre of the wall, ensuring that the end cut panels are of equal size. The optimum aesthetic effect is obtained by installing the panels in broken bond.

- Position and fix the first line of KN brackets along the wall base batten at 600mm centres, coinciding with the position of the panels, using two screws (3.5 x 25mm) per bracket. Place the first line of Troldtekt panels along the base of the wall by carefully sliding the panel edge grooves on to the KN brackets and fix the next line of KN brackets continuously as you install panels. It is important that the panels are installed tightly together. Fix the Troldtekt panels along the wall, lengthwise, using Troldtekt screws for wood (4.2 x 55mm) 25mm from the edges. The maximum distance between the screws should be 600mm. Be careful not to overdrive the screws and so cause an indentation in the panel.

- The final line of panels at the top of the wall are fastened along their top edges using Troldtekt screws for wood (4.2 x 55mm for 35mm panel) positioned 25mm from panel edges at maximum 600mm centres. Be careful not to overdrive the screws and so cause indentations in the panels.

- If required place 30 or 50mm acoustic pads behind Troldtekt panels and fit tightly between battens fixing where necessary.

940 BOARD MOISTURE CONTENT AND CONDITIONING

- Moisture content of boards at time of fixing: Appropriate to end use.

- It is important that the Troldtekt panels are allowed to acclimatise prior to installation. Acclimatisation means that the panels must adapt to the temperature and humidity in the room where they are to be installed.